# DRAFT: Information About the App, Sensors, and Alerts

### We have sensors that measure environmental variables in the soil and in the air. In addition to moisture/temperature/EC of soil, we also measure temperature, humidity, CO2, and Total Volatile Organic Compounds (TVOC) of air, and the amount of light that vines are receiving. All of this information be used for early detection of frost, mold, and black rot among other benefits. Note that the measurements are made every 15 minutes, reported to the cloud for processing, and the farmer can see the results for all measurements and analytics on her/his phone and desktop. Note that the smallest interval allowed for graphs is 30 minutes to avoid synchronization issues on the graph (the 15-minute reporting by each sensor box may happen at different times). Here are the automatically selected graph-data intervals for the date ranges selected by the user:

### Range <= 2 days: 30-minute (default)

### Range > 2 and <= 5 days: 1 hour

### Range > 5 and <= 90 days: 1 day

### The app currently only saves data for the last three months.

### Reducing Disease and Frost Risk

The soil sensor measures temperature, soil moisture, and electrical conductivity. These three values can be used to describe water, plant stress, and soil chemistry:

| Sensor | What it really tells us |
| --- | --- |
| Soil moisture | Water availability + irrigation effectiveness |
| Soil temperature | Root activity + growth speed + disease risk |
| Electrical conductivity (EC) | Nutrients + salinity + fertilizer dynamics |

More information is provided in the following paragraphs that allows farmers to take actions based on these measurements.

1) Soil Moisture (a real important variable)

Irrigation decisions involve knowing when and how much to irrigate. Moisture trends tell us:

- When plants start experiencing water stress

- Whether irrigation actually reached root depth

- Whether water is draining too fast (waste) or staying too long (root rot)

Practical actions:

- Irrigate only when soil reaches crop-specific depletion level

- Detect over-watering early

- Validate irrigation system uniformity across zones

- Prevent irrigation right before rain (combine with weather later)

Example of actions to be taken: If moisture drops fast after irrigation → sandy soil → water drains fast → shorter, more frequent watering. However, if moisture stays high for days → heavy clay → risk of root disease → reduce irrigation. This alone saves water, energy, pump wear, and fertilizer runoff.

2) Soil Temperature (plant metabolism clock)

Soil temperature controls root activity, not air temperature. Plants don’t care what the weather app says since roots live underground. Soil temperature reveals:

- Germination timing

- Nutrient uptake speed

- Frost risk at root zone

- Disease likelihood

Planting timing

Each crop has a soil temperature threshold for planting time:

| Crop | Soil temp needed |
| --- | --- |
| Corn | ~10°C / 50°F |
| Soybeans | ~12°C / 54°F |
| Grapevines | ~16°C / 60°F |

Plant too early leads to seeds rotting instead of germinating; plant too late will lead to yield loss. Sensors and our app remove guesswork for the farmer.

Fertilizer timing

Nutrient uptake rises sharply once soil warms. Fertilizing cold soil will lead to pollution and wasted resources.

Frost protection

Sudden drops in soil temp can warn before plant damage happens, thus, triggering introduction of frost irrigation, frost candles, fans, and/or covers.

3) Electrical Conductivity (fertilizer and salinity reality check)

This one gets ignored constantly, which is impressive given how expensive fertilizer is. Electrical Conductivity (EC) measures dissolved salts in soil water. That includes:

- Nitrogen

- Potassium

- Other nutrients

- Salinity buildup from irrigation

So, EC is a nutrient concentration indicator. For fertilizer optimization, combine EC and soil moisture information from sensors:

If moisture increases and EC drops → nutrients diluted → plants absorbing → fertilization working.
If EC spikes after fertilization and stays high → over-fertilization → risk of root burn and wasted money. Farmers routinely apply fertilizer blindly; EC lets them see the nutrient lifecycle.

Salinity monitoring (huge in irrigated farms)

Over time irrigation leaves salts behind.

High EC + low moisture = salt concentration → plant stress → yield loss.

Sensor lets farmer:

- Detect salt buildup early

- Schedule leaching irrigation to flush salts

- Avoid long-term soil degradation

This protects land value and, thus, is economically critical.

4) The real power: combining the three

Individual information from soil sensors is useful but together they become diagnostics. Here are four examples of diagnostic patterns:

Case A — Soil moisture low + Soil temp high + EC high
Plants stressed + water evaporating + salts concentrating → Urgent irrigation needed.

Case B — Soil moisture high + EC high
Too much fertilizer or poor drainage → Risk of root damage.

Case C — Soil moisture good + EC dropping
Plants actively absorbing nutrients → Growth phase confirmed.

Case D — Soil moisture high + Soil temp low
Cold wet soil → Perfect conditions for fungal disease.

5) Yield forecasting & precision agriculture

With historical information provided over time in an effective app, the farmer gets:

- Soil behavior per field zone

- Irrigation zone tuning

- Fertilizer rate optimization

- Detection of equipment failures

- Reduced input costs and higher yield consistency

With the soil sensors measuring moisture, temperature, and EC, we have a decision engine. Farmers move from: “I think the field needs water and fertilizer” to: “The west zone needs 20% less irrigation and no nitrogen this week.” This would be very helpful to save time and money for farmers.

### Alerts for Spring Frost, Mold, and Black Rot

Among the biggest concerns for farmers in the Midwest, we can name the onset of frost (in May or April), mold, and black rot. Grapes are fragile and Midwest weather is basically a mood swing factory. Spring frost roulette, humid summers, fungal apocalypse. To address these challenges, you now have a mini weather station + soil lab at your disposal that runs every 15 minutes.

Let’s focus on the three nightmares: spring frost, mold, black rot. Midwest vineyards sit in a humid continental climate:

- Wet springs

- Humid summers

- Rapid temperature swings

- Frequent dew formation

Which means fungal disease pressure is naturally high and frost risk is real into May. Your sensors basically let you detect microclimate events inside the farm, which matter a lot more than regional forecasts.

PART 1 — EARLY FROST DETECTION (April–May panic season)

Spring frost kills new shoots. One bad night leads to massive yield loss. Frost happens when:

- Clear skies

- Calm wind

- Rapid night cooling

- High humidity → dew → freezing

The sensors let us detect radiation frost before it hits.

Critical frost indicators from the sensors

Frost risk in April and May skyrockets when:

- Temp drop rate > 2°F per hour after sunset

- No cloud cover (we can infer from light sensor)

- Humidity rising to 90% and temperature is in the 30s

When air temp falls at a fast rate, relative humidity rises; once RH approaches 90–100%, dew forms. Dew formation means the air is about to hit the dew point, and after that, freezing could happen fast. Soil temperature could also be a frost severity predictor. Warm soil releases heat at night. Cold soil does not. If soil temp is:

- Above ~50°F → soil buffers frost risk

- Below ~45°F → frost risk increases dramatically

So, frost risk is highest when: Night cooling + high humidity + cold soil. That is our Frost Risk Index. This combo is a frost warning 2–4 hours before damage. That gives time to:

- Turn on wind machines

- Start frost irrigation

- Activate heaters / Frost candles

- Send workers alerts

The app allows the farmer to set alerts and be notified based on these conditions.

PART 2 — MOLD & BLACK ROT

These diseases don’t appear randomly. They require a very specific microclimate conditions, and your sensors basically measure the ingredients.

Key fact: Fungal diseases need leaf wetness + warmth + humidity duration.

You don’t have a leaf wetness sensor yet, but we will add that later.

Fungi explode when:

- Warm temperature

- High humidity

- Prolonged moisture on plant surfaces

The sensors can infer all three.

Mold (Powdery & Downy Mildew)

Powdery mildew loves these conditions:

- 70–85°F air temp

- Humidity > 60%

- Shade / low light

- Moderate moisture

But it hates:

- Direct sunlight

- Dry air

Early detection pattern

If the app detects repeated days in summer and early fall with:

- Air temp: 68–86°F

- Humidity > 65% for > 6 hours/day

- Light levels low inside canopy

- Soil moisture consistently high

The vines are in Powdery Mildew Incubation Mode. The disease is forming before visible symptoms. This is the moment to spray preventatively and not wait until after the leaves look ugly. Downy mildew / general mold loves these conditions:

- Humidity > 85%

- Nighttime moisture

- Wet leaves overnight

- Warm days after wet nights

Early detection pattern

Night:

- Humidity > 90%

- Temp 55–70°F
Morning:

- Slow temperature rise

- Low light (cloudy morning)

This means dew persisted for hours. Wet leaves for 6–10 hours = fungal party. The sensors can detect dew duration indirectly: Night humidity near 100% + flat temp curve = leaves stayed wet. This predicts infection events days before symptoms.

PART 3 — BLACK ROT (vineyard villain)

Black rot requires:

- Warm temps (70–85°F)

- Rain or prolonged leaf wetness

- Several hours of moisture

It often starts after spring rains.

To create a black rot early-warning system, we look for this sequence in the data from sensors:

Event day

- Soil moisture jumps (rain)

- Humidity > 90%

- Air temp 70–85°F

Following 48 hours

- Warm humid days continue

That combo is basically the disease’s origin story. This is when fungicide timing matters most.
Miss the window and infection already happened.

PART 4 — How the farmer should actually use the app

The app doesn’t show only raw graphs; it flags risk windows:

Frost alert

- Fast evening cooling

- Humidity rising >90%

- Soil cold

Mold risk alert

- Humidity high for long periods

- Warm temps

- Low light inside canopy

Black rot alert

- Rain → warm humid period

This turns data into action timing:

- Frost protection night

- Fungicide spray timing

- Irrigation reduction to reduce humidity

In practice, this sensor system and app give the vineyard owner:

- 3–6 hour frost early warning

- Disease risk prediction days before symptoms

- Optimal fungicide timing

- Reduced spraying frequency

- Lower crop loss

- Higher grape quality

All these advantages from a few sensors reporting the air and soil conditions every 15 minutes is amazing!